OsmoFlux — Production Data Freeze Manifest

SYNOPSYS 2026  ·  Rohan Agarwal

Freeze date: 2026-03-07  ·  Notebook: OsmoFlux.ipynb

# 1. Purpose

This document records the exact data artifacts, experiment specifications, and reproduction instructions for every figure displayed on the OsmoFlux ISEF poster. It is intended to allow any judge, reviewer, or auditor to independently verify the data behind each finding and to exactly reproduce any figure from scratch.

For each figure: (1) the source experiment ID and run timestamp are given, (2) the SHA-256 hash of the source data file is recorded so the file can be verified as unmodified, (3) the complete set of model parameters is listed, and (4) step-by-step reproduction instructions are provided.

# 2. Poster Figure → Data Map

| Fig # | Title | Experiment ID | Finding |
| --- | --- | --- | --- |
| 1 | Architecture crossover — power vs β | CASCADE-CL-SWEEP (20260301_145354) | Finding 1: Fresh beats serial, 240/240 configs, up to +168% |
| 2 | Compliance gap — fresh vs serial | CASCADE-CL-SWEEP (20260301_145354) | Finding 2: 96–100% vs 66–100% harm reduction, 34 pp gap |
| 3 | Fouling amplifies cascade advantage (Jebel Ali) | CASCADE-OPT-JebelAli-SEVERE | Finding 3: 312% clean → 376% severe, +64 pp amplification |
| 4 | Morris sensitivity scatter | MORRIS-SCREENING | Methodology: fouling ranks last in absolute sensitivity |
| 5 | MC-TEA NPV — Moderate fouling | BRINE-CARLSBAD-MC (20260305_152413) | Finding 4: P(NPV>0) = 37.0% under moderate fouling |
| 6 | MC-TEA NPV — Severe fouling | BRINE-CARLSBAD-MC-SEVERE (20260303_210416) | Finding 4: P(NPV>0) = 12.0% under severe fouling |
| 7 | BE3 — Cascade advantage map | BREAKEVEN-CARLSBAD | Discussion: economic breakeven boundary |
| 8 | D3 — Compliance threshold sweep | CASCADE-THREE-CONFIG | Discussion: N_total scaling of compliance |

# 3. Data Artifacts & SHA-256 Hashes

The following files are the canonical frozen data artifacts. SHA-256 hashes were computed on 2026-03-07. Any file with a matching hash is byte-for-byte identical to the production data.

| Experiment | File | SHA-256 |
| --- | --- | --- |
| BRINE-CARLSBAD-MC (20260305_152413) | mc_tea.csv | 96ac9598ebbc4fd6f8eaa0dbb9e1dd41385b83e660967c55e7100658ad5fcd88 |
| BRINE-CARLSBAD-MC-SEVERE (20260303_210416) | mc_tea.csv | 73922e48497e3956cd7e442f82d16334cbc8f4d7d369990393709b7fd49d4e41 |
| CASCADE-CL-SWEEP (20260301_145354) | cl_sweep_all.parquet | — verify from runs/ directory on host machine |
| MORRIS-SCREENING (latest run) | morris_screening.csv | — verify from runs/ directory on host machine |
| CASCADE-OPT-JebelAli-SEVERE (latest) | cascade_sweep.parquet | — verify from runs/ directory on host machine |
| BREAKEVEN-CARLSBAD (latest) | breakeven_grid.parquet | — verify from runs/ directory on host machine |
| CASCADE-THREE-CONFIG (latest) | three_config_all.parquet | — verify from runs/ directory on host machine |

Note: To verify a file: python3 -c "import hashlib; print(hashlib.sha256(open('file','rb').read()).hexdigest())"

# 4. Complete Experiment Specifications

## Figures 1 & 2: CASCADE-CL-SWEEP

Experiment ID: CASCADE-CL-SWEEP  |  Run timestamp: 20260301_145354

Source file: runs/CASCADE-CL-SWEEP/20260301_145354/cl_sweep_all.parquet

Generated by: run_architecture_comparison() → run_cascade_sweep() per (C_L, architecture) pair

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.15 (Carlsbad Poseidon SWRO brine) |
| C_L_IN_MOL_L sweep | [0.02, 0.10, 0.50] (treated WW / impaired / ambient seawater) |
| Q_H_M3_S | 5.0e-6 m³/s |
| Q_L_M3_S | 5.0e-6 m³/s |
| GAP_M | 2.0e-4 m |
| DIVALENT_FRACTION | 0.20 |
| FOULING_MODEL | none (base); swept via fouling_beta_vals |
| FOULING_BETA sweep | [0.0, 0.2, 0.4, 0.6, 0.8] |
| FOULING_C_REF_MOL_L | 0.5 |
| FOULING_N_EXP | 0.5 |
| A_MEM_M2 | 0.05 m²/pair |
| N_total | 120 cell pairs total |
| N_stages sweep (s_vals) | [1, 2, 3, 4, 5, 6, 8, 10] |
| N_per_stage sweep (n_vals) | [3, 5, 10, 15, 20, 30, 40, 60, 120] |
| Architectures | fresh-dilute (True) and serial pass-through (False) |
| Solver | model_1d_axial_series_current (1d_series) |
| MODE | max_power |
| N_SEG | 40 axial segments |

### Reproduction Instructions — Figures 1 & 2

1. Open OsmoFlux.ipynb and run all cells up to and including cell 18 (function definitions).

2. In cell 21, ensure ACTIVE_CASCADE_IDS contains 'CASCADE-ARCHITECTURE-COMPARISON'.

3. Confirm _CASCADE_BASE_OVERRIDES matches the table above.

4. Confirm _CL_SCENARIOS = [0.02, 0.1, 0.5].

5. Run cell 21. Output written to runs/CASCADE-CL-SWEEP/<timestamp>/.

6. Run cell 37 to regenerate poster figures from the new parquet.

Note: Figure 1 uses crossover_power_CH1p15_POSTER.png; Figure 2 uses abstract_fig2_compliance_gap.png.

## Figure 3: CASCADE-OPT-JebelAli-SEVERE

Experiment ID: CASCADE-OPT-JebelAli-SEVERE  |  Run timestamp: latest in runs/

Source file: runs/CASCADE-OPT-JebelAli-SEVERE/<timestamp>/cascade_sweep.parquet

Generated by: run_cascade_sweep(cascade_jebel, save=True, make_plots=True)

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.80 (Jebel Ali Arabian Gulf SWRO) |
| C_L_IN_MOL_L | 0.04 (brackish groundwater) |
| Q_H_M3_S | 5.0e-6 m³/s |
| Q_L_M3_S | 5.0e-6 m³/s |
| GAP_M | 2.0e-4 m |
| DIVALENT_FRACTION | 0.35 (RO brine enrichment, Giwa 2017) |
| FOULING_MODEL | exp_outlet (constant outlet-heavy biofilm) |
| FOULING_BETA sweep | [0.0, 0.2, 0.4, 0.6, 0.8, 1.0, 1.2, 1.5] |
| FOULING_C_REF_MOL_L | 0.5 |
| FOULING_N_EXP | 0.5 |
| A_MEM_M2 | 0.05 m²/pair |
| N_stages sweep (s_vals) | [1, 2, 3, 4, 5, 6, 8, 10] |
| N_per_stage sweep (n_vals) | [3, 5, 10, 20, 40] |
| Architecture | Fresh-dilute (fresh_dilute=True) |
| Solver | model_1d_axial_series_current (1d_series) |
| MODE | max_power |
| N_SEG | 40 axial segments |
| Note on dual fouling | exp_outlet has hardcoded beta=0.40 always active. FOULING_BETA sweep adds CDF (inlet-heavy divalent scaling). Figure shows cascade GAIN only (%) — not absolute power — to avoid dual-fouling artifact. |

### Reproduction Instructions — Figure 3

1. Run all cells up to cell 18.

2. In cell 33, change 'if False:' to 'if True:' to enable the Jebel Ali severe fouling block.

3. Run cell 33. Output written to runs/CASCADE-OPT-JebelAli-SEVERE/<timestamp>/.

4. Run cell 37 to regenerate abstract_fig3_fouling_amplifies_cascade.png.

Key: Figure plots cascade gain (%) vs FOULING_BETA for S=10, N=40/stage. Endpoint annotations: 312% (clean, β=0) and 376% (severe, β=1.5), +64 pp amplification.

## Figure 4: MORRIS-SCREENING

Experiment ID: MORRIS-SCREENING  |  Run timestamp: latest in runs/

Source file: runs/MORRIS-SCREENING/<timestamp>/morris_screening.csv

| Parameter | Value |
| --- | --- |
| Base conditions | C_H=1.15, C_L=0.02, N=10, A=0.05 m², β=0.6, f_div=0.20 |
| Solver | 1d_series, max_power, N_SEG=40 |
| Morris method | r=10 trajectories, p=4 levels, seed=42 |
| N parameters | 12 |
| Parameter ranges | C_H:[0.5,2.0], C_L:[0.005,0.1], N:[5,40], A:[0.005,0.05], α:[0.92,0.99], ASR_CEM:[2e-4,5e-4], ASR_AEM:[2.5e-4,6e-4], GAP:[1e-4,4e-4], β:[0.0,1.0], f_div:[0.0,0.30], Q_H:[5e-6,5e-5], Q_L:[5e-6,5e-5] |
| Total model calls | ~130 (r × (k+1) = 10 × 13) |
| Key result | μ*(β)=0.045 — fouling ranks last in absolute power sensitivity |

### Reproduction Instructions — Figure 4

1. Run all cells up to cell 20.

2. Ensure 'MORRIS-SCREENING' is in ACTIVE_EXPERIMENT_IDS in cell 21.

3. Run cell 21. Output written to runs/MORRIS-SCREENING/<timestamp>/.

4. Run cell 37 to regenerate morris_sensitivity_scatter.png.

## Figures 5 & 6: MC-TEA NPV — Moderate and Severe Fouling

CRITICAL NOTE ON NPV FORMULA:

Both figures use: NPV = NET_ANNUAL_USD × annuity(DISCOUNT_RATE, T=20) − PLANT_CAPEX_USD

For Figure 5 (moderate): NET_ANNUAL_USD is recomputed by adding back ANNUAL_MEM_REPLACE_USD before applying the formula. This corrects a cost-model inconsistency between the two runs (SEV run predates membrane-lifetime sampling). Both arms are thus on an identical cost basis.

### Figure 5: BRINE-CARLSBAD-MC (Moderate Fouling)

Run timestamp: 20260305_152413  |  n=5,000 samples  |  seed=42

Source file: runs/BRINE-CARLSBAD-MC/20260305_152413/mc_tea.csv

SHA-256: 96ac9598ebbc4fd6f8eaa0dbb9e1dd41385b83e660967c55e7100658ad5fcd88

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.15 (Carlsbad) |
| C_L_IN_MOL_L | 0.03 |
| N_CELL_PAIRS | 80 (harm_sweep deployment optimal) |
| A_MEM_M2 | 0.05 m²/pair |
| DIVALENT_FRACTION | 0.20 |
| FOULING_MODEL | none (base); β sampled per MC draw |
| FOULING_BETA sampling | Uniform(0.0, 0.8) |
| ELEC_PRICE sampling | Lognormal(μ=0.22, σ=30%) $/kWh |
| BRINE_DISP_COST sampling | Uniform(0.04, 0.25) $/m³ |
| CAPEX_PER_M2 sampling | Triangular(30, 50, 100) $/m² |
| DISCOUNT_RATE sampling | Uniform(3%, 8%) |
| NPV horizon | T=20 years |
| PLANT_BRINE_FLOW | 190,000 m³/day |
| Verified P(NPV>0) | 37.0% (after removing ANNUAL_MEM_REPLACE_USD) |
| P50 NPV | −$26M |

### Figure 6: BRINE-CARLSBAD-MC-SEVERE (Severe Fouling)

Run timestamp: 20260303_210416  |  n=5,000 samples  |  seed=400

Source file: runs/BRINE-CARLSBAD-MC-SEVERE/20260303_210416/mc_tea.csv

SHA-256: 73922e48497e3956cd7e442f82d16334cbc8f4d7d369990393709b7fd49d4e41

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.15 (Carlsbad) |
| C_L_IN_MOL_L | 0.02 |
| N_CELL_PAIRS | 80 |
| A_MEM_M2 | 0.05 m²/pair |
| DIVALENT_FRACTION | 0.35 (RO brine enrichment, Giwa 2017) |
| FOULING_MODEL | exp_outlet |
| FOULING_BETA | 1.5 (fixed — Vermaas 2013 severe regime) |
| ELEC_PRICE sampling | Uniform(0.08, 0.35) $/kWh |
| BRINE_DISP_COST sampling | Uniform(0.04, 0.25) $/m³ |
| CAPEX_PER_M2 sampling | Triangular(30, 50, 100) $/m² |
| DISCOUNT_RATE sampling | Uniform(3%, 8%) |
| NPV horizon | T=20 years |
| PLANT_BRINE_FLOW | 190,000 m³/day |
| Verified P(NPV>0) | 12.0% (stored NPV_USD used directly) |
| P50 NPV | −$86M |

### Reproduction Instructions — Figures 5 & 6

1. Run all cells up to cell 20.

2. For moderate: cell 20 defines BRINE-CARLSBAD-MC. Add it to ACTIVE_EXPERIMENT_IDS in cell 21 and run.

3. For severe: In cell 35, change 'if False:' to 'if True:'. Run cell 35.

4. Run cell 37 to regenerate poster_MC_npv_moderate_vs_severe.png.

Cell 37 will automatically: (a) load mc_tea from the latest run, (b) add back ANNUAL_MEM_REPLACE_USD for moderate, (c) compute corrected P(NPV>0), (d) save figure.

5. Verify: terminal should print 'Finding (4): 37.0% -> 12.0% under severe fouling'.

## Figure 7: BREAKEVEN-CARLSBAD — Cascade Advantage Map (BE3)

Experiment ID: BREAKEVEN-CARLSBAD  |  Run timestamp: latest in runs/

Source file: runs/BREAKEVEN-CARLSBAD/<timestamp>/ub_fig_BE3_advantage_map.png

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.15 (Carlsbad) |
| C_L_IN_MOL_L | 0.02 |
| N_total | 120 (N_stages=6, N_per=20) |
| A_MEM_M2 | 0.05 m²/pair |
| Q_H_M3_S | 5.0e-6 m³/s |
| ELEC_PRICE | $0.22/kWh (fixed, Carlsbad market rate) |
| DISCOUNT_RATE | 3.9% (fixed) |
| NPV horizon | T=20 years |
| OPEX fraction | 3% of capex/year |
| CAPACITY_FACTOR | 0.913 |
| CAPEX_VALS sweep ($/m²) | [2, 5, 8, 10, 15, 20, 30, 40, 50, 60, 75, 100] |
| DISP_VALS sweep ($/m³) | linspace(0.02, 0.25, 60) |
| BETA_VALS sweep | [0.0, 0.2, 0.4, 0.6, 0.8] (averaged for BE3 map) |
| PLANT_BRINE_FLOW | 190,000 m³/day |

### Reproduction Instructions — Figure 7

1. Run all cells up to cell 18.

2. Run cell 44 (BREAKEVEN-CARLSBAD). Output written to runs/BREAKEVEN-CARLSBAD/<timestamp>/.

3. Run cell 37 to copy ub_fig_BE3_advantage_map.png to runs/poster_BE3_cascade_advantage_map.png.

## Figure 8: CASCADE-THREE-CONFIG — Compliance Threshold Sweep (D3)

Experiment ID: CASCADE-THREE-CONFIG  |  Run timestamp: latest in runs/

Source file: runs/CASCADE-THREE-CONFIG/<timestamp>/three_config_all.parquet

| Parameter | Value |
| --- | --- |
| C_H_IN_MOL_L | 1.15 (Carlsbad) |
| C_L_IN_MOL_L | 0.03 |
| A_MEM_M2 | 0.05 m²/pair |
| Q_L_M3_S | 5.0e-6 m³/s (primary scenario) |
| N_total sweep | [160, 200, 240, 300] cell pairs total |
| DIVALENT_FRACTION | 0.35 |
| FOULING_MODEL | exp_outlet |
| FOULING_BETA sweep | [0.0, 0.2, 0.4, 0.6, 0.8, 1.0, 1.2, 1.5] |
| Architectures | single-stack, serial cascade, fresh-dilute cascade |
| Solver | 1d_series, max_power, N_SEG=40 |

### Reproduction Instructions — Figure 8

1. Run all cells up to cell 18.

2. In cell 21, ensure 'CASCADE-THREE-CONFIG' is active.

3. Run cell 21 (or run cell 39 directly if parquets already exist).

4. Run cell 37 to regenerate discussion_fig_D3_compliance_threshold_sweep.png.

# 5. Judge Verification Procedure

To verify any specific finding during judging:

Step 1 — Verify data files are unchanged:

python3 -c "import hashlib; print(hashlib.sha256(open('mc_tea.csv','rb').read()).hexdigest())"

Compare output to SHA-256 values in Section 3 above.

Step 2 — Verify key statistics from the CSV directly:

python3 -c "import pandas as pd, numpy as np

df = pd.read_csv('BRINE-CARLSBAD-MC__20260305_152413__mc_tea.csv')

r = df['DISCOUNT_RATE']; ann = (1-(1+r)**-20)/r

npv = (df['NET_ANNUAL_USD']+df['ANNUAL_MEM_REPLACE_USD'])*ann - df['PLANT_CAPEX_USD']

print('P(NPV>0) moderate:', round(100*(npv>0).mean(), 1), '%')"

Step 3 — Regenerate any figure:

Open OsmoFlux.ipynb in Jupyter, follow reproduction instructions in Section 4, run cell 37.

# 6. Honest Model Limitations

| Limitation | Impact and Defense |
| --- | --- |
| No water osmosis | Slightly underestimates driving force recovery per stage. Strengthens, not weakens, the cascade argument. |
| Fixed permselectivity (α=0.975) | Real α drops to 0.90–0.92 at C_H=3.0. Affects absolute predictions ~5–10% but both architectures shift together; comparative conclusions hold. |
| Isothermal (25°C) | Real seawater 15–35°C gives ~5–8% power variation. Shifts all scenarios equally. |
| Spacer shadow factors (not explicit Sherwood) | Vermaas 2014 factors (β_sol=1.56, β_mem=1.10) measured under conditions including boundary layer effects. Conservative. |
| MC-TEA: mem replacement cost excluded (moderate) | ANNUAL_MEM_REPLACE_USD present in MOD run but absent in SEV run. Excluded from both for apples-to-apples comparison. Documented in cell 37 comment and this manifest. |
| T_opt = 365 days (never clean) | Cleaning costs exceed recovered power at all tested cost levels. Correct strategy: design for fouling tolerance. |
| Dilute source logistics not costed | Fresh-dilute cascade requires co-located low-salinity source. Carlsbad has Encina site. Serial cascade is fallback — performs worse but avoids this constraint. |

OsmoFlux — Production Data Freeze Manifest  ·  ISEF 2026  ·  Rohan Agarwal, Blue Leaf Labs  ·  Mentor: Garcia